- PANEL HEATER

- INSTRUCTION MANUAL

Instruction for Ceiling Mounting Way A:

Only if below ceiling mounting sets included.

X 2 SETS

- Drill two holes on the ceiling with correct distance A and distance

B.

| Panel Size/mm | Hole Distances of Screws/mm |
| --- | --- |
| 605*605 (or similar) | A310*B240mm |
| 705*605 (or similar) | A360*B240mm |
| 805*605 (or similar) | A480*B240mm |
| 905*605 (or similar) | A480*B240mm |
| 1005*605 (or similar) | A480*B240mm |
| 1205*605 (or similar) | A660*B240mm |
| 1205*755 (or similar) | A660*B390mm |
| 1205*805 (or similar) | A660*B390mm |
| 1205*905 (or similar) | A660*B470mm |
| 1205*1005 (or similar) | A660*B470mm |

- Mount the two hanging units onto ceiling with screws, and turn

the screws to the end tightly, this is very important.

- Take off all the four plastic caps from rear of the panel

- Slide the panel to make ceiling hanging units into one side of the

panel rear bracket first, then push the whole panel close to ceiling

and slide the panel back, to make the other end of the ceiling

hanging unit enters the other side.

(View after mounting finished)